# Methodology for the Multi-Scale Remote Sensing Approach for Area Under Cane Estimation.

Table of Contents

Methodology for the Multi-Scale Remote Sensing Approach for Area Under Cane Estimation.	1

1	Executive Summary	4

1.1	Introduction and Problem Statement	4

1.1.1	The Challenge	4

1.1.2	Challenges faced during the study	4

1.1.3	Proposed Solution Overview	4

2	Scientific Foundation and Justification of the proposed methodology	5

2.1	Can drone derived Vegetation Indices (VI) be used to train Satellite Imagery?	5

2.2	Beyond Vegetation Indices: Full spectral feature approach	6

2.3	Addressing Spectral Confusion with Similar Crops	7

2.3.1	Temporal Phenology Analysis	7

2.3.2	Structural and Textural Discrimination	8

2.3.3	Biophysical Parameter Inversion	8

2.3.4	Crop Calendar Integration	8

3	Comprehensive Methodology	9

3.1	Phase 1: Drone Data Acquisition and Processing	9

3.1.1	Sampling Design	9

3.1.2	Drone Flight Planning	9

3.1.3	Image Processing Workflow	11

3.2	Phase 2: Reference Data Creation	11

3.2.1	Manual Digitization Protocol	11

3.2.2	Training Sample Extraction	12

3.3	Phase 3: Sentinel-2 Data Acquisition and Preprocessing	12

3.3.1	Data Selection	12

3.3.2	Atmospheric Correction	12

3.3.3	Cloud and Shadow Masking	12

3.3.4	Geometric Co-registration	13

3.3.5	Feature Engineering	13

3.4	Phase 4: Machine Learning Classification	13

3.4.1	Algorithm Selection	13

3.4.2	Training Strategy	14

3.5	Phase 5: Classification and Area Calculation	14

3.5.1	Large-Scale Classification	14

3.5.2	Post-Classification Refinement	14

3.5.3	Vector Conversion	15

3.5.4	Area Calculation	15

3.5.5	Hierarchical Area Summaries	16

3.6	Phase 6: Validation and Accuracy Assessment	16

3.6.1	Independent Validation Dataset	16

3.6.2	Field Validation Protocol	16

3.6.3	Accuracy Assessment	17

3.6.4	Error Analysis	17

# Executive Summary

Ecospace Services Limited’s proposed approach of using drone-derived vegetation indices to train Sentinel-2 imagery for area under estimation is scientifically valid and operationally feasible. This report provides a detailed methodology combining high-resolution drone mapping with medium-resolution satellite imagery through supervised classification and machine learning techniques. The approach addresses Kenya Sugar Board's budget constraints while maintaining accuracy through strategic sampling and validation protocols.

Key Findings:

- Drone-derived spectral signatures can effectively train Sentinel-2 classification models through proper spatial aggregation and feature engineering.

- Multiple spectral features beyond vegetation indices improve classification accuracy (85-92% achievable).

- Crop phenology timing and multi-temporal analysis are critical for separating sugarcane from spectrally similar crops.

- A stratified sampling approach requiring approximately 30% drone coverage of the cane growing regions can validate and calibrate area estimates cost-effectively.

## Introduction and Problem Statement

### The Challenge

The Kenya Sugar Board (KSB) requires consistent and accurate sugarcane area estimation for:

- Yield prediction and planning

- Resource allocation

- Policy formulation

- Industry monitoring and evaluation

### Challenges faced during the study

- High-resolution satellite imagery procurement delays

- Budget limitations for extensive GNSS ground surveys

- Need for cost-effective, scalable solutions

- Requirement for verification protocols

### Proposed Solution Overview

A hierarchical remote sensing approach that:

- Uses limited drone coverage with DJI Mavic 3 Multispectral as high-resolution reference data

- Extracts comprehensive spectral signatures from drone imagery

- Trains machine learning classifiers on Sentinel-2 imagery

- Maps entire sugarcane growing regions

- Implements stratified validation protocols

# Scientific Foundation and Justification of the proposed methodology

## Can drone derived Vegetation Indices (VI) be used to train Satellite Imagery?

The principle of spectral signature transferability across sensors is well-established in remote sensing science, provided proper calibration and spatial scaling are applied.

Key Conditions for Success:

- Spectral Band Alignment

DJI Mavic 3M multispectral bands vs. Sentinel-2:

| Band | DJI Mavic 3M | Sentinel-2 | Alignment |
| --- | --- | --- | --- |
| Green | 560±16 nm | 560 nm (B3) | Excellent |
| Red | 650±16 nm | 665 nm (B4) | Excellent |
| Red Edge | 730±16 nm | 705 nm (B5), 740 nm (B6) | Good |
| NIR | 860±26 nm | 842 nm (B8) | Excellent |

The spectral alignment is sufficient for signature transfer.

- Spatial Aggregation Theory

When aggregating high-resolution drone pixels to Sentinel-2 resolution (10m), the relationship follows:

S10m =  1ni=1nDi+ ε

Where:

- S₁₀ₘ = Sentinel-2 10m pixel value

- Dᵢ = Individual drone pixels within 10m area

- n = Number of drone pixels in 10m cell

- ε = Sensor-specific noise and atmospheric effects

- Vegetation Index Consistency

NDVI calculated from both sensors:

NDVI = (NIR - Red) / (NIR + Red)

Studies have shown a correlation coefficients of 0.85 - 0.95 between drone and satellite-derived NDVI for agricultural crops when properly calibrated. (Pádua et al., 2019; Crusiol et al., 2020).

## Beyond Vegetation Indices: Full spectral feature approach

We used a comprehensive spectral feature, and not just vegetation indices because VI tend to compress multidimensional spectral information into single values, losing discriminative power needed to separate spectrally similar crops.

- Raw Spectral Bands

- Blue, Green, Red, Red Edge, NIR, SWIR1, SWIR2

- Each band captures unique biophysical properties

- Vegetation Indices Suite

- NDVI (Normalized Difference Vegetation Index)

NDVI = (NIR - Red) / (NIR + Red)

Measures photosynthetic activity and biomass

- EVI (Enhanced Vegetation Index)

EVI = 2.5 × [(NIR - Red) / (NIR + 6×Red - 7.5×Blue + 1)]

Reduces atmospheric and soil background effects

- NDRE (Normalized Difference Red Edge)

NDRE = (NIR - RedEdge) / (NIR + RedEdge)

Sensitive to chlorophyll content, useful for crop discrimination

- SAVI (Soil Adjusted Vegetation Index)

SAVI = [(NIR - Red) / (NIR + Red + L)] × (1 + L)

Where L = 0.5 for moderate vegetation cover

- NDWI (Normalized Difference Water Index)

NDWI = (Green - NIR) / (Green + NIR)

Measures vegetation water content

- GCVI (Green Chlorophyll Vegetation Index)

GCVI = (NIR / Green) - 1

Correlates with leaf chlorophyll content

- Texture Features (from Grey Level Co-occurrence Matrix)

- Contrast, Homogeneity, Entropy, Correlation

- Computed on NIR and Red bands

- Captures spatial patterns unique to sugarcane rows

- Temporal Features

- Mean, Standard Deviation, Max, Min across growing season

- Phenological metrics (green-up date, peak greenness, senescence)

## Addressing Spectral Confusion with Similar Crops

Maize, sorghum, Napier grass, and other grasses can have similar spectral signatures to sugarcane at certain growth stages. We therefore propose to use the multi-dimensional discrimination analysis using:

### Temporal Phenology Analysis

Key Insight unique phenological profile for Sugarcane are:

- Maturity Period: 12-18 months.

- Perennial characteristics

- Distinct harvest patterns

- Biomass Accumulation

- Continuous canopy cover

- Tillering

Temporal Discrimination Approach:

- Multi-Temporal Composite

Collect Sentinel-2 imagery across 12 months with 10-day intervals (36 images minimum).

For each pixel, calculate:

Temporal Signature = [NDVI₁, NDVI₂, ..., NDVI₃₆, σ_NDVI, Amplitude, Phase]

Where:

- σ_NDVI = Standard deviation of NDVI over time

- Amplitude = Maximum (NDVI) – Minimum (NDVI)

- Phase = Time of maximum NDVI

- Phenological Metrics

Extract using TIMESAT or similar algorithms:

- Start of Season (SOS) - When NDVI exceeds threshold

- Peak of Season (POS) - Maximum NDVI timing

- End of Season (EOS) - When NDVI drops below threshold

- Length of Season (LOS) = EOS - SOS

- Seasonal Amplitude = Peak NDVI - Base NDVI

- Seasonal Integral = Area under NDVI curve

Sugarcane Characteristics:

- LOS > 300 days (vs. maize: 90-120 days)

- Low amplitude variation (continuous growth)

- Multiple ratoon cycles visible over 2-3 years

### Structural and Textural Discrimination

Row Structure Analysis:

Gray Level Co-occurrence Matrix (GLCM) It is a statistical method for extracting texture features from images. Instead of looking only at pixel values (like reflectance), it looks at how pixel values co-occur with their neighbours in a defined direction and distance.

Sugarcane has distinctive row patterns visible in 10m resolution at a certain stage of growth:

Texture Feature = GLCM (NIR_band, distance=1, angles= [0°, 45°, 90°, 135°])

Metrics:

- Contrast: Higher in sugarcane due to row-inter row differences

- Homogeneity: Lower than uniform crops

- Correlation: Exhibits directional pattern

### Biophysical Parameter Inversion

Canopy Structure Modelling:

Use PROSAIL radiative transfer model to estimate:

LAI (Leaf Area Index) = f(ρ_red, ρ_NIR, ρ_RedEdge)

CWC (Canopy Water Content) = f(ρ_SWIR)

Cab (Chlorophyll content) = f(ρ_RedEdge)

Sugarcane-specific ranges:

- LAI: 4-8 (mature crop)

- CWC: Higher than most annual crops

- Cab: Moderate to high

### Crop Calendar Integration

Incorporating the local agricultural calendar knowledge.

Implementation:

- Map known sugarcane harvest schedules

- Identify expected planting windows

- Cross-reference with temporal profiles

- Flag anomalies for manual verification

Rule-based refinement:

IF (LOS > 300 days) AND (Harvest_signature detected) AND (Location in sugarcane zone):

Confidence_Score += 0.3

# Comprehensive Methodology

## Phase 1: Drone Data Acquisition and Processing

### Sampling Design

A stratified random sampling approach was used to ensure that drone data captured the diversity of sugarcane fields across different regions and conditions.

The number of samples required was determined using the following formula:

n = (Z² × σ² × N) / (E² × (N-1) + Z² × σ²)

Where:

- n = required sample size

- Z = Z-score for desired confidence (1.96 for 95%)

- σ = estimated population variation (0.2 for sugarcane presence)

- N = total number of 10m pixels in the study area

- E = acceptable margin of error (5%)

To ensure good representation, drone surveys covered 20–30% of the total sugarcane area. Sampling included:

- Multiple mill zones (3–5 areas minimum)

- Fields of different crop ages (new, mid-growth, harvest-ready)

- Various topographies (flat, rolling, sloped terrain)

- Different farm sizes, from smallholder plots to large estates

### Drone Flight Planning

Drone flights were carefully planned to ensure consistent image quality and coverage.

Flight parameters included:

- Altitude: 100 m above ground (Ground Sampling Distance ≈ 5 cm)

- Image overlap: 80% forward, 70% side

- Flight speed: 5 m/s

- Optimal time: Within 2 hours before or after solar noon (sun angle 45–60°)

- Weather: Clear skies with wind speed less than 5 m/s

Coverage expectations:

- Per battery: 15–20 hectares

- Per day (per team): 100–150 hectares under optimal conditions

### Image Processing Workflow

Drone images were processed using DJI Terra or Agisoft Metashape to produce orthomosaics and elevation models.

- Photogrammetric Processing Steps

- Import and align images

- Generate a sparse point cloud

- Build a dense point cloud

- Create Orthomosaic for each spectral band

- Derive a Digital Elevation Model (DEM)

Quality control metrics:

- Reprojection error < 1 pixel

- Ground Control Point (GCP) RMSE < 0.1 m

- Point density > 400 points/m²

- Geometric Co-registration

The final drone Orthomosaic were aligned to Sentinel-2 satellite geometry with a registration error less than 5 m to ensure accurate integration with satellite data.

RMSE_registration < 0.5 × Sentinel_pixel_size (< 5m)

## Phase 2: Reference Data Creation

### Manual Digitization Protocol

High-resolution drone orthomosaics were used to manually digitize field boundaries in QGIS 3.x.

Digitization rules included:

- Minimum mapping unit (MMU): 0.1 ha (1,000 m²)

- Clear boundary delineation along actual field edges

- Exclude roads, buildings, and water bodies

- Classify fields into:

  - Pure sugarcane (>90% coverage)

  - Mixed sugarcane (60–90%)

  - Other crops

  - Non-agricultural areas

Quality control:

- Two independent operators digitized the same areas.

- Discrepancies were reviewed by an expert.

- 10% of polygons were verified in the field.

### Training Sample Extraction

The digitized reference data were converted into training samples matching Sentinel-2’s 10m pixel size. For each pixel, average reflectance was computed from the drone data to represent its spectral signature. Only “pure” sugarcane pixels (where >80% of the pixel was sugarcane) were selected to ensure high-quality training data.

Selection criteria:

- Spectral homogeneity (Coefficient of Variation < 20%)

- Balanced number of samples per class

- Minimum spacing of 50 m between samples (to reduce spatial autocorrelation)

Target sample sizes:

- Sugarcane: 2,000–3,000 pixels

- Other crops: 1,500–2,000 pixels per class

- Non-crop areas: ~1,000 pixels

## Phase 3: Sentinel-2 Data Acquisition and Preprocessing

### Data Selection

Sentinel-2 imagery was collected for at least 12 months (preferably 24 months) to capture multiple crop cycles.

Selection criteria:

- Cloud cover < 10%

- Revisit frequency: every 10 days (combining Sentinel-2A and 2B)

- Coverage: all sugarcane-growing regions

### Atmospheric Correction

Atmospheric correction was applied using Sen2Cor or ACOLITE to convert top-of-atmosphere reflectance into surface reflectance using the equation:

ρ_surface = (ρ_TOA - ρ_path) / (τ × τ_sun)

Validation was performed by comparing corrected values with ground spectral measurements or known reference surfaces (e.g., water or bare soil).

### Cloud and Shadow Masking

To ensure clean data, clouds and shadows were detected and removed using a multi-stage process:

- Use Sentinel-2’s built-in Scene Classification Layer (SCL)

- Apply additional spectral thresholds:

  - Clouds: (Blue > 0.3) AND (NDSI < 0.8) AND (NDVI < 0.6)

  - Shadows: (NIR < 0.15) AND (SWIR < 0.12)

- Apply temporal interpolation to fill missing data from nearby clear-date images.

### Geometric Co-registration

All Sentinel-2 images were aligned within a common coordinate system (UTM Zone 36S/37S for Kenya).
Registration errors were maintained below 0.5 pixel (5 m) for precise pixel-level consistency across time.

### Feature Engineering

A comprehensive set of features was generated for classification:

- Spectral Bands (10 total):
Blue (B2), Green (B3), Red (B4), Red Edge (B5–B7), NIR (B8, B8A), SWIR (B11, B12).

- Vegetation Indices (12+):
NDVI, EVI, SAVI, NDRE, NDWI, GCVI, ARVI, SIPI, GNDVI, etc.

- Texture Features (16 total):
Gray-Level Co-occurrence Matrix (GLCM) metrics calculated from the NIR band in 4 directions (0°, 45°, 90°, 135°).

- Temporal Statistics:
Mean, median, standard deviation, min, max, percentiles (25th, 75th), and coefficient of variation per pixel.

- Phenological Metrics (8 total):
Derived from vegetation time-series, including:

- Start, peak, and end of season (SOS, POS, EOS)

- Length of season (LOS)

- Growth rate, senescence rate, amplitude, and seasonal integral

Overall, each pixel had approximately 150–200 features describing its spectral, temporal, and structural characteristics.

## Phase 4: Machine Learning Classification

### Algorithm Selection

The classification was performed using Random Forest (RF) and Gradient Boosting (XGBoost) algorithms.

These were chosen because they:

- Handle large, complex datasets effectively

- Are robust against overfitting and noise

- Provide variable importance rankings

- Are computationally efficient for large-scale mapping

- Deliver higher accuracy compared to traditional classifiers

### Training Strategy

- Data Splitting

The dataset was divided as follows:

- 60% for training

- 20% for validation (hyperparameter tuning)

- 20% for testing (final accuracy assessment)

Spatial stratification ensured that samples from all mill zones were represented in each split.

- Cross-Validation

A 5-fold spatial cross-validation was used to assess model stability:

- The study area was divided into 5 geographic blocks.

- Each model was trained on 4 blocks and validated on the 5th, rotating across folds.

- Final accuracy was calculated as the average across all folds.

- Handling Class Imbalance

Where sugarcane samples were fewer than other land cover types:

- Class weights were adjusted inversely to class frequency.

- Synthetic Minority Over-sampling Technique (SMOTE) was tested.

- Stratified sampling ensured balanced representation across classes.

## Phase 5: Classification and Area Calculation

### Large-Scale Classification

Because the study area covers a wide region, the analysis is carried out in smaller manageable tiles. Each tile measures approximately 10 km × 10 km, with a 500 m overlap to ensure that features near tile edges are not lost or misclassified.

The processing is performed in parallel (multiple tiles processed simultaneously) to save time and increase efficiency. For each tile, image features (e.g., spectral bands, vegetation indices, texture) are extracted and classified using the trained model. The model predicts the land cover type for every pixel within that tile, and the classified outputs are saved individually.

### Post-Classification Refinement

After classification, several refinement steps are applied to clean and improve the accuracy of the map:

- Majority Filter

This filter helps remove the “salt-and-pepper” noise often seen in pixel-based classification.
Each pixel’s class is re-evaluated within a 3×3 or 5×5 window, and the pixel is assigned the majority class within that neighbourhood. This step smooths the map and produces more uniform patches.

- Minimum Mapping Unit (MMU)

To avoid including very small, insignificant patches, a Minimum Mapping Unit (MMU) is applied. In this project, the MMU is set to 0.1 hectares, which corresponds to 10 pixels at 10 m resolution. All patches smaller than this threshold are removed or merged with neighbouring areas to maintain a realistic and interpretable map.

- Boundary Smoothing

Some field boundaries may appear jagged due to pixel-level variations. A boundary smoothing (simplification) step is applied using a tolerance of about 5 meters.
This generalization creates smoother and more natural-looking field boundaries while retaining their overall shape.

### Vector Conversion

Once the classification is finalized, the raster output (pixel-based) is converted into vector polygons. Each polygon represents a distinct classified field or land cover unit.

During this conversion, several attributes are attached to each polygon, including:

- Unique ID (identifier for each field)

- Area (in hectares)

- Perimeter

- Mean NDVI value during the peak growing season

- Classification confidence (probability or certainty of correct classification)

- Mill zone (if applicable)

- District or County

This vector dataset forms the core of spatial statistics and reporting.

### Area Calculation

For each polygon, the area is calculated using a projected coordinate system (UTM) to ensure measurements are in true metric units.

- Area (m²) is computed directly from the polygon geometry.

- It is then converted to hectares (ha) by dividing by 10,000.

- The total sugarcane area is obtained by summing the areas of all classified sugarcane polygons.

### Hierarchical Area Summaries

The results are aggregated at different administrative and operational levels:

- Total area by Class and or mill zone

- Area by district or county

- Area by farm size category

- Area by crop age, where possible

## Phase 6: Validation and Accuracy Assessment

Once the classification maps are generated, it is essential to validate their accuracy using independent reference data. This phase ensures that the mapped results truly represent the real-world crop conditions.

### Independent Validation Dataset

To evaluate how well the classification model performed, a stratified random sampling approach is used to select validation points. This ensures fair representation of all classes and regions across the study area.

- Sample size: 50,000 –100,000 validation points

- Stratification criteria: Based on both predicted class (e.g., sugarcane, fallow, other crops) and geographic zones (districts or mill zones).

Ground truth sources include:

- GNSS field visits:  50,000 to 100,000 parcels to be collected directly on-site using high-accuracy GPS receivers.

- High-resolution imagery interpretation: 50,000 to 100,000 parcels to be identified using drone or commercial satellite images.

- Farmer interviews and records: 100,000 to 200,000 parcels to be verified through discussions with local farmers and production records.

These combined sources provide a balanced and reliable validation dataset.

### Field Validation Protocol

Field validation should be performed systematically to ensure high-quality reference data. The field teams should be equipped and trained to collect standardized information from each sampled point.

Equipment to be Used:

- Handheld GNSS receiver (positional accuracy < 1 m)

- Digital camera or mobile phone for geotagged photographs

- Ecofield Mobile data collection app (for field forms and uploads).

At each validation location, the following data will be recorded:

- Geographic coordinates (latitude/longitude or UTM)

- Confirm crop type (e.g., sugarcane, maize, fallow, mixed)

- Estimated crop age or growth stage

- Four geotagged photos (N, E, S, W directions)

- Notes on anomalies such as mixed cropping or recent harvesting

These observations are later used to cross-check and validate the classification output.

### Accuracy Assessment

After compiling the ground truth dataset, a confusion matrix will be created to compare predicted classes with observed (true) classes.

Key accuracy metrics be calculated include:

- Overall Accuracy (OA): Percentage of correctly classified points

- Producer’s Accuracy (PA): Reliability of each mapped class from the reference perspective

- User’s Accuracy (UA): Likelihood that a mapped class truly represents what it claims

- R Squared Coefficient (R2): Statistical measure of agreement beyond random chance

### Error Analysis

To better understand misclassifications, the errors will be analysed both statistically and spatially:

- Systematic Biases: Identify whether certain classes (e.g., young cane vs. mature cane) are consistently over- or under-predicted.

- Spatial Distribution of Errors: Map where most errors occur—along boundaries, in shaded areas, or mixed crop zones.

- Error by Crop Age or Terrain: Analyze accuracy variations due to topography, soil brightness, or crop phenology.